# Column dictionary

Dataset: design_points_results_combined.csv — 900 designs × 47 columns. Two-stage Miller-compensated op-amp, UMC 130 nm, VDD = 1.2 V. Column names follow Table 2 of the paper; all values are stored in SI units.

Roles

- Target (independent): the 11 parameters predicted by every model (green).

- Target (dependent): 8 parameters set by the topology constraints and copied from their matched devices (grey).

- Input: specification: 25 performance quantities (white).

- Input: operating point: 3 gm/ID values from the sizing flow and lookup charts (yellow).

- Derived: 6 features computed at training time and not stored in the CSV (blue), giving 34 model inputs in total.

Sign convention: idd, Power consumption and Area/IDD are stored as negative values (current drawn from VDD).

| Column (CSV header) | Table 2 name | Role | Unit | Source | Notes |
| --- | --- | --- | --- | --- | --- |
| L1 | L1 | Target (independent) | m | Sizing flow | Input-pair length (M1, M2) |
| L2 | L2 | Target (dependent) | m | Sizing flow | = L1 (topology constraint) |
| L3 | L3 | Target (independent) | m | Sizing flow | PMOS load length (M3, M4) |
| L4 | L4 | Target (dependent) | m | Sizing flow | = L3 (topology constraint) |
| L5 | L5 | Target (independent) | m | Sizing flow | Tail / bias length (M5) |
| L6 | L6 | Target (dependent) | m | Sizing flow | = L3 (sizing method) |
| L7 | L7 | Target (dependent) | m | Sizing flow | = L5 (sizing method; M7 mirrors M5) |
| L8 | L8 | Target (dependent) | m | Sizing flow | = L5 (topology constraint) |
| W1 | W1 | Target (independent) | m | Sizing flow | Input-pair width (M1, M2) |
| W2 | W2 | Target (dependent) | m | Sizing flow | = W1 (topology constraint) |
| W3 | W3 | Target (independent) | m | Sizing flow | PMOS load width (M3, M4) |
| W4 | W4 | Target (dependent) | m | Sizing flow | = W3 (topology constraint) |
| W5 | W5 | Target (independent) | m | Sizing flow | Tail width (M5) |
| W6 | W6 | Target (independent) | m | Sizing flow | Second-stage input width (M6, PMOS) |
| W7 | W7 | Target (independent) | m | Sizing flow | Second-stage load width (M7, NMOS) |
| W8 | W8 | Target (dependent) | m | Sizing flow | = W5 (topology constraint) |
| IBIAS | I_BIAS | Target (independent) | A | Sizing flow | Bias current into diode-connected M8 |
| CC | C_C | Target (independent) | F | Sizing flow | Miller compensation capacitor |
| Rz | R_z | Target (independent) | Ohm | Sizing flow | Nulling resistor; formula chosen per design ('infinity' or 'pole-zero', Fig. 4) |
| CL | C_L | Input: specification | F | Specified | Load capacitance |
| Gain | A_vd | Input: specification | dB | Cadence (AC) | Differential DC gain |
| PM | PM | Input: specification | deg | Cadence (AC) | Phase margin |
| PM_freq | PM frequency | Input: specification | Hz | Cadence (AC) | Frequency at which PM is measured |
| UGBW | UGBW | Input: specification | Hz | Cadence (AC) | Unity-gain bandwidth |
| GBW | GBW | Input: specification | Hz | Specified; Eq. (3) | Gain-bandwidth product, g_m1/(2*pi*C_C) |
| BW | BW | Input: specification | Hz | Cadence (AC) | -3 dB bandwidth |
| Gain Margin | Gain margin | Input: specification | dB | Cadence (AC) | Gain Margin |
| GM_freq | GM frequency | Input: specification | Hz | Cadence (AC) | Frequency at which gain margin is measured |
| PSR_dB | PSR | Input: specification | dB | Cadence (AC) | Power-supply rejection |
| Av_cm | A_vcm | Input: specification | dB | Cadence (AC) | Simulated common-mode gain. |
| vmin | v_min | Input: specification | V | Cadence (transient) | Lower voltage level of the transient output step, used to compute the slew rate. |
| vmax | v_max | Input: specification | V | Cadence (transient) | Upper voltage level of the transient output step, used to compute the slew rate. |
| SR | SR | Input: specification | V/s | Cadence (transient) | Slew rate (1 V/us = 1e6 V/s) |
| idd | I_DD | Input: specification | A | Cadence (transient) | Supply current; stored negative (current drawn from V_DD) |
| Power consumption | Power consumption | Input: specification | W | Cadence | = V_DD * idd (V_DD = 1.2 V); stored negative, same sign convention as idd |
| dc_offset | DC offset | Input: specification | V | Cadence (DC) | Signed |
| vout_max | v_out,max | Input: specification | V | Cadence (DC) | Stored to nearest 10 mV |
| vout_min | v_out,min | Input: specification | V | Cadence (DC) | Stored to nearest 1 mV |
| swing | Swing | Input: specification | V | Cadence (DC) | = vout_max - vout_min (within 5 mV rounding) |
| RzCC corner frequency | RzCC corner frequency | Input: specification | — | Calculated | 12πRzCc |
| gm/id (input) | g_m/I_D of input pair | Input: operating point | S/A (1/V) | Sizing flow and lookup charts | Input pair M1, M2 |
| gmoverid(load) | g_m/I_D of load | Input: operating point | S/A (1/V) | Sizing flow and lookup charts | PMOS load M3, M4 |
| gmoverid(tail) | g_m/I_D of tail | Input: operating point | S/A (1/V) | Sizing flow and lookup charts | Tail M5 |
| Intrinsic speed | Intrinsic speed | Input: specification | Hz | Cadence (DC) | Speed capability of the transistors/stages before the Miller capacitor C_C limits the overall amplifier bandwidth. |
| (vout_max - vout_min)/VDD | swing/V_DD | Input: specification | dimensionless | Calculated | = swing / 1.2 V |
| Area | Area | Input: specification | m^2 | Calculated | Sum of W*L over the eight transistors |
| Area/IDD | Area/I_DD | Input: specification | m^2/A | Calculated | = Area / idd; negative because idd is stored negative |
| PM/UGBW | (derived feature, Section 4) | Derived (computed at training; not stored in CSV) | deg/Hz | Preprocessing | PM / UGBW |
| GBW/UGBW | (derived feature, Section 4) | Derived (computed at training; not stored in CSV) | dimensionless | Preprocessing | GBW / UGBW |
| log10(GBW) | (derived feature, Section 4) | Derived (computed at training; not stored in CSV) | log10(Hz) | Preprocessing | log10 of GBW |
| log10(UGBW) | (derived feature, Section 4) | Derived (computed at training; not stored in CSV) | log10(Hz) | Preprocessing | log10 of UGBW |
| GBW^2 | (derived feature, Section 4) | Derived (computed at training; not stored in CSV) | Hz^2 | Preprocessing | GBW squared |
| UGBW^2 | (derived feature, Section 4) | Derived (computed at training; not stored in CSV) | Hz^2 | Preprocessing | UGBW squared |